# EXP-6: INDUSTRIAL HVAC CONTROL FIRMWARE: ADC TEMPERATURE ACQUISITION AND TIMER-INTERRUPT PWM FAN CONTROL

# CODE:

#define TEMP A0

#define FAN  8

volatile byte pwm = 0;

volatile byte count = 0;

ISR(TIMER1_COMPA_vect)

{

count++;

if (count >= 100)

count = 0;

digitalWrite(FAN, count < pwm);

}

void setup()

{

Serial.begin(9600);

pinMode(FAN, OUTPUT);

TCCR1A = 0;

TCCR1B = (1 << WGM12) | (1 << CS11);

OCR1A = 199;

TIMSK1 = (1 << OCIE1A);

sei();

}

void loop()

{

int adc = analogRead(TEMP);

float temp = adc * 500.0 / 1023.0;

if (temp < 25)

pwm = 0;

else if (temp < 30)

pwm = 30;

else if (temp < 35)

pwm = 60;

else

pwm = 90;

Serial.print("Temperature: ");

Serial.print(temp, 1);

Serial.print(" C  PWM: ");

Serial.print(pwm);

Serial.println("%");

delay(1000);

# }

# OUTPUT:

# REFLECTION: I learned how to interface the LM35 temperature sensor with Arduino UNO and acquire temperature using ADC. I understood how to control an HVAC fan using timer interrupts and PWM based on temperature. This experiment improved my knowledge of Embedded C programming and Proteus simulation.